Metadata: PTSP Winter 0708 Data.xls 


METADATA

Winter 2007/2008 Dataset

Putney Study Plot (PTSP)

Senator Beck Basin Study Area (SBBSA), San Juan Mountains 

I.  Putney Study Plot (PTSP)

1) Site

a) Putney Study Plot (PTSP)

i) RF Station ID = 1

(1) 1 hour data arrays:  Flag field = 101

(2) 3 hour data arrays: Flag field = 103

(3) 24 hour data arrays: Flag field = 124

b) Site unlocked; datalogger enclosure locked with keyed padlock

c) Lat/Lon: 37( 53' 34" N, 107( 41' 42" W; NAD 1927

d) Elevation: 12,325 feet, 3,757 meters

e) Aspect: level 

f) Slope Angle: 0 degrees

g) Photographs: http://www.snowstudies.org/senbeck_sa.html
h) Changes to site: no alterations to tundra site since original installation aside from removal of obsolete equipment from mast and cleanup of power and data land lines

i) Comments: PTSP utilizes a 30' steel mast installed on private property in the early 1970's; site used with the gracious permission of property owner Tronox, LLC.

2) Operation Status

a) Winter 2007/2008 dataset from 0000 hrs October 1, 2007 (day-of-year 274) through 2400 hrs June 20, 2008 (DOY 172).

b) All data collected on Mountain Standard Time, by day-of-year (DOY)

i) System clocks not switched from/to Mountain Daylight Time in Fall or Spring

ii) 24 hour summary datasets (Flag = 224) are calendar day 

c) Measurements are calculated in both SI (International System of Units) and imperial/US units to facilitate use by other public safety agencies.

3) Type of Site

a) Study Plot: elevated and prominent 'sub-summit' location on N/S ridgeline.

b) PTSP site is private property; permission to operate PTSP kindly granted to CSAS by TRONOX, LLC.

4) Power

a) Solar powered by 20 watt photovoltaic panel providing regulated charge to two 90 amp hour gel 12V DC batteries.

5) Sensors

a) Wind Speed and Direction 

i) Properties

(a) Make: RM Young

(b) Model: #05103-5 

(c) Serial Number: WM61999

(d) Type: Wind Monitor

ii) Installation

(a) Height above ground: 9.5 m

(b) Distance from tower or obstacle: 0.5 m

(c) Date originally installed at PTSP: October 4, 2004  (DOY 278); 

(d) Service/calibration dates: new instrument at installation; original calibration July 9, 2004. 

(e) Comment: mounted to arm extending due north from mast

(f) Comment: Campbell Scientific Edlog software instruction P69, parameter 3 set at option 2 to calculate both ‘resultant’ and ‘scalar’ mean wind speed, calculate ‘resultant’ mean wind direction (only), standard deviation of mean wind direction.  Instruction P73 calculates peak wind gust.   

b) Air Temperature and Humidity

i) Properties

(a) Make: Campbell/Vaisala 

(b) Model: HMP-35C

(c) Serial Number: 7063

(d) Type: Vaisala humiter and Fenwall UUT51J1 thermistor

ii) Installation

(a) Height above ground: 3.1 m

(b) Date originally installed at PTSP: December 11, 2004 (DOY 346)

(c) Service/calibration dates: serviced and recalibrated by Campbell Scientific November, 2004.
(d) Comments: mounted on east side of mast at 3.1 m above ground to prevent damage from blowing gravel/rock/sand. 

(e) Comments: suspicious minimum temperature value at 2100 hrs on January 28, 2008 (DOY 28) at -48.07 C (-54.53 F).  Wind speeds averaged 23 m/s during that hour, from 300 degrees, a very high average, and gusted to 33 m/s.   

6) Datalogger

a) Make: Campbell Scientific

b) Model: CR10X

c) Serial Number: 40362

d) Type: fully programmable measurement and control system with ring memory

e) Acquisition date: September, 2004

f) Date originally installed at PTSP: October 3, 2005 (DOY 277)

g) Service/calibration dates: original calibration certificate September 16, 2004

7) Data Retrieval

a) Radio telemetry using phone-to-RF base station; no repeater utilized

i) Campbell Scientific model RF 310M modem; serial #6928

ii) Model RF310 Maxon SD-125 V2 VHF radio: serial #030604688

iii) Cellwave PS1121-2 dipole antenna 

8) Software

a) Campbell Scientific LoggerNet 2.1c

b) One PTSP Edlog (LoggerNet 2.1c) version (V1) utilized during Winter 2007/2008. 

9) Observer Contact Information

a) Name:  Chris Landry - Executive Director

b) Organization:  Center for Snow and Avalanche Studies

c) Address:  PO Box 190, Silverton, CO, USA 81433

d) Telephone:  (970) 387-5080

e) Email:  clandry@snowstudies.org
f) Website: http://www.snowstudies.org/index.html



10) Data processing and output methods (data points as labeled in Excel Spreadsheet):

a) Measurement execution interval:  5 seconds

b) Measurements: see table below 

Data processing and output methods (as labeled in Excel workbook PTSP Winter 0708 Data.xls):

	Spreadsheet Labels 

(some abbreviations expanded)
	Loc'n

#
	LoggerNet 2.1c

Ouput Instruction
	Type of

Measurement
	Notes

	Flag
	1
	P80
	na
	201 = 1 hour array

203 = 3 hour array

212 = solar noon array

224 = 24 hour array

	Year
	2
	P77
	Na
	Calendar year 

	Day of Year
	3
	P77
	na
	Sequential day of calendar year

	Time MST
	4
	P77
	na
	Mountain Standard Time at end of array period, in military time; previous day at midnight, 2400 at midnight

	Min Air C 

Time Minimum
	5

6
	P74
	Minimize
	Array minimum air temperature (C), time of minimum

	Max Air C

Time Maximum
	7

8
	P73
	Maximize
	Array maximum air temperature (C), time of maximum

	Air F Min
	9
	P74
	Minimize
	Array minimum air temperature (F)

	Air F Max
	10
	P73
	Maximize
	Array maximum air temperature (F)

	RH
	11
	P70
	Sample
	Samples relative humidity (%) once at end of array

	Peak Gust MPH 

Time Peak Gust

Peak Gust MS

S(Scalar) Avg Wd MPH

U(Resultant) Avg Wd MPH

Avg Wind Dir'n

Std Dev Dir'n

S(Scalar) Avg Wd MS

U(Resultant) Avg Wd MS

Avg Wind Dir'n

Std Dev Dir'n
	12

13

14

15

16

17

18

19

20

21

22
	P73

P69
	Maximize

Wind vector
	Array peak gust in MPH, MS

P69 output option '2': scalar mean horizontal wind speed S; resultant mean wind speed U; resultant (speed-weighted) mean wind direction; standard deviation wind direction using Campbell Scientific algorithm.

	Sys Volts
	23
	P70
	Sample
	Voltage at datalogger (prior to regulation) sampled once at end of array 

	Day Avg Air C
	24
	P71
	Average
	Averages 17,280 measurements of air temperature collected during a day, from 0000 hrs through 2400 hrs

	Day Avg Air F
	25
	P71
	Average
	Averages 17,280 measurements of air temperature collected during a day, from 0000 hrs through 2400 hrs


Page 3 of 3

